Rev 2 Edger white board wiring rev 0.52 11052022

Examine all the photos first to understand part orientations. Notice how wires are dressed out away from the Espressif board so the unused pins can be accessed with jumper wires.

Completed board:

Parts Placement:

Breadboard column number		Device						Pin number

63					Espressif ESP32-C3-DEVKITC-02 		1

38					Adafruit 1764 JTAG USB breakout		“5V”

32					Renesas ForgeFPGA SLG47004V-DIP	1

40					Blue LED					Anode (long lead)

40					1k resistor					-

37					Yellow LED					Anode (short lead)

37					1K resistor					-

Espressif ESP32-C3-DEVKITC-02 board PHYSICAL PINS:

Length	(cm)	Color		From			To

2.5		Blk		30 “G”			Breadboard ground rail (blue line)

2.5		Red		18 “3.3V”		Breadboard power rail (red line)

8.0		Org		14 “0”			FPGA physical pin 8 “SCL”

8.1		Ylw		13 “1”			FPGA physcial pin 9 “SDA”

7.7		Wht		4  “18”	JTAG		JTAG USB D- (Adafruit breakout)

8.3		Gry		5  “19” 		JTAG USB D+ (Adafruit)

2.5		Blk		15 “G”			Ground rail (blue line)

Adafruit 1764 USB breakout:

5.0		Blk		to	Ground rail (blue line) JTAG USB Gnd

Renesas ForgeFPGA SLG47004V-DIP PHYSICAL PINS:

2.5		Red		to	Power rail (red line)	 FPGA pin 1 “VDD”

2.5		Blk		to	Ground rail (blue line) FPGA pin 11 “GND”

5.9		Vio		to	FPGA pin 6 “GPIO 15” FPGA pin 16 “GPIO 16”

Breadboard:

6.8		Red		Power rail 1 (red line)  to	Power rail 2 (red line)